|  | Threat Description |  | Explosives Mass (TNT equivalent) | Building Evacuation Distance | Outdoor Evacuation Distance |
| --- | --- | --- | --- | --- | --- |
| High Explosives (TNT Equivalent) |  | Pipe Bomb | 5 lbs 2.3 kg | 70 ft 21 m | 850 ft 259 m |
|  |  | Suicide Belt | 10 lbs 4.5 kg | 90 ft 27 m | 1,080 ft 330 m |
|  |  | Suicide Vest | 20 lbs 9 kg | 110 ft 34 m | 1,360 ft 415 m |
|  |  | Briefcase/Suitcase Bomb | 50 lbs 23 kg | 150 ft 46 m | 1,850 ft 564 m |
|  |  | Compact Sedan | 500 lbs 227 kg | 320 ft 98 m | 1,500 ft 457 m |
|  |  | Sedan | 1,000 lbs 454 kg | 400 ft 122 m | 1,750 ft 534 m |
|  |  | Passenger/Cargo Van | 4,000 lbs 1,814 kg | 640 ft 195 m | 2,750 ft 838 m |
|  |  | Small Moving Van/ Delivery Truck | 10,000 lbs 4,536 kg | 860 ft 263 m | 3,750 ft 1,143 m |
|  |  | Moving Van/Water Truck | 30,000 lbs 13,608 kg | 1,240 ft 375 m | 6,500 ft 1,982 m |
|  |  | Semitrailer | 60,000 lbs 27,216 kg | 1,570 ft 475 m | 7,000 ft 2,134 m |
|  | # Threat Description |  | LPG Mass/Volume1 | Fireball Diameter | Safe Distance |
| Liquefied Petroleum Gas (LPG - Butane or Propane) |  | Small LPG Tank | 20 lbs/5 gal 9 kg/19 l | 40 ft 12 m | 160 ft 48 m |
|  |  | Large LPG Tank | 100 lbs/25 gal 45 kg/95 l | 69 ft 21 m | 276 ft 84 m |
|  |  | Commercial/Residential LPG Tank | 2,000 lbs/500 gal 907 kg/1,893 l | 184 ft 56 m | 736 ft 224 m |
|  |  | Small LPG Truck | 8,000 lbs/2,000 gal 3,630 kg/7,570 l | 292 ft 89 m | 1,168 ft 356 m |
|  |  | Semitanker LPG | 40,000 lbs/10,000 gal 18,144 kg/37,850 l | 499 ft 152 m | 1,996 ft 608 m |

Based on the maximum amount of material that could reasonably fit into a container or vehicle. Variations possible.

Governed by the ability of an unreinforced building to withstand severe damage or collapse.

Governed by the greater of fragment throw distance or glass breakage/falling glass hazard distance. These distances can be reduced for personnel wearing ballistic protection. Note that the pipe bomb, suicide belt/vest, and briefcase/suitcase bomb are assumed to have a fragmentation characteristic that requires greater standoff distances than an equal amount of explosives in a vehicle.

Assuming efficient mixing of the flammable gas with ambient air.

Determined by U.S. firefighting practices wherein safe distances are approximately 4 times the flame height. Note that an LPG tank filled with high explosives would require a significantly greater standoff distance than if it were filled with LPG.